# Introduction

The Astra SR MCU from Synaptics is a cutting-edge, high-performance microcontroller designed to bring advanced AI and vision capabilities to a wide range of applications. The Astra SR MCU seamlessly integrates powerful processing, sophisticated vision systems, and exceptional energy efficiency into a compact package.

Harness the power of artificial intelligence with the Astra SR MCU family, featuring scalable multi-core architectures tailored for both AI-enabled and non-AI use cases. AI-capable variants integrate an Arm® Cortex®-M55 CPU running up to 400 MHz alongside an Ethos™-U55 micro NPU, delivering robust performance for real-time inferencing and tiered processing.

The Astra MCU portfolio also includes SL2610-based configurations with a Cortex-M52 (CM52) running up to 200 MHz, designed for high-performance control and signal-processing workloads without AI acceleration.

Across the Astra MCU family, the platform is equipped with a comprehensive set of interfaces to communicate with system components such as the application processor (AP), camera sensors, digital microphones, and other peripherals, enabling flexible integration across a wide range of embedded applications.

The Astra MCU SDK 1.6.0 is a software development kit that supports the SR110 and SL2610 MCUs. It provides a comprehensive collection of source code, tools, and documentation, and is built on CMSIS. The SDK can be compiled using GCC, Arm Compiler 6 (AC6), and LLVM for SR110, while SL2610 supports compilation using GCC only. The high-level components included in this SDK are described below.

- Free RTOS

- Source codes of various peripheral drivers, image processing, low power sensing blocks.

- SR110-specific reference applications demonstrating camera-based sensing and wake-up, person detection, classification, pose detection, segmentation, and hand gesture recognition. It also features JPEG pre-roll with frame transfer over USB CDC to a host PC or via SPI to an SPI-capable receiver, along with audio processing use cases supported on the SR110 platform.

- Astra MCU SDK VSCode Extension that supports build and image generation, along with integrated video streaming capabilities.

- Related SDK Documents

# General

| Date | 09 August 2026 |
| --- | --- |
| Version number | 1.6.0 |
| Supported SoCs | SR110 Rev. B (B0) FCCSP & WLCSP SL2610 |
| Supported Platforms | SR110 RDK Board Rev A/B/C/D/E SL2610 RDK Board Rev B/C |
| File name |  |
| APBL | SR110: APBL v012F SL2610: 1.6.0 |
| SPK | SR110: SPK RC4.0 |
| VS Code | 1.5.0 |

# Supported Peripherals

| HW Component | CM55-B0 | CM52 |
| --- | --- | --- |
| UART | √ | √ |
| IOMUX | √ | √ |
| GPIO | √ | √ |
| DMA | √ Memory to Memory | √ Memory to Memory |
| I2C-S/M | √ | √ |
| Cache | √ | √ |
| Watchdog | √ | √ |
| Clocks | √ | √ |
| Timers | √ | √ |
| LP-Sense | √ | √ |
| USB 2.0 Device | √ | √ |
| SPI-S/M | √ | √(SPI-S) |
| I2S M | √ | √ |
| xSPI (1/4 bits) | √ | √ |
| MIPI CSI RX -0 | √ | X |
| MIPI CSI RX -1 | X | X |
| DVP Serial | √ | X |
| DVP Parallel | X | X |
| SDIO | √ | X |
| SPWM | X | √ |
| I3C | X | √ |
| CAN | X | √ |
| PDM | X | √ |
| ADC | X | √ |
| Flight Control | X | √ |

# Supported Software Components

| SW Component | CM55-B0 | CM52 |
| --- | --- | --- |
| Free RTOS | √ | √ |
| DMEM Service | √ | √ |
| Logger Service | √ | √ |
| USB Tiny USB Stack | √ | √ |
| Flash Services | √ | √ |
| Host API Service | √ | √ |

# 5.  Supported Power Modes

| Mode | CM55-B0 | CM52 |
| --- | --- | --- |
| Active | Full Operational mode: Gear1 (G1), Gear2 (G2), Gear3 (G3) Mode Entry: boot process Mode Exit: N/A | Active Sleep: TBD |
| Hibernation | All memories: in retention Gear 3: (HP Proc) - disabled Gear 2: (LP Proc) - disabled Gear 1: (LP Sense) – disabled Gear 0 (AON): awake Mode entry: Host API’s power manager commands (0x1-0x4) Mode exit: internal/external event (timer, GPI) | Deep Sleep partial retention: TBD Deep Sleep Mem retention: TBD Standby: TBD |

# VS Code Extension

| SW Tool | CM55-B0 | CM52 |
| --- | --- | --- |
| Code Development | √ | √ |
| Debug | √ | √ |
| Image Generator | √ | √ |
| Image Flashing | √ | √ |
| Video Streamer | √ | X |

# Reference Applications

| Use Case | CM55-B0 w/ Ethos U55 | CM52 |
| --- | --- | --- |
| Vision: Person Detection | √ | X |
| Vision: Person Classification | √ | X |
| Vision: Person Pose Detection | √ | X |
| Vision: Person Segmentation | √ | X |
| Vision: Hand Gesture Detection | √ | X |
| Vision: Pre-roll | √ | X |
| Vision: Face ID + Hand Gesture Detection | √ | X |
| Hand Gesture + Air Mouse | √ | X |

# Sample Applications

| Example | CM55-B0 w/ Ethos U55 | CM52 |
| --- | --- | --- |
| Inference - Single SRAM NN Model | √ | X |
| Inference - Single Flash NN Model | √ | X |
| UART Sample Application with flow control | √ | X |
| I2C Sample Application | √ | √ |
| PINMUX Sample Application | √ | X |
| GPIO Sample Application | √ | X |
| CLK Sample Application | √ | X |
| SPI Sample Application | √ | X |
| SDIO Sample Application (SDCARD & SDIO devices) | √ | X |
| FATFS Sample Application | √ | X |
| BT Sample Application | √ | X |
| System Manager | X | √ |
| DMA Memory to Memory Sample Application | √ | √ |
| SPWM Application | X | √ |
| I2S/TDM Application | X | √ |
| ADC Application | X | √ |
| CAN | X | √ |

# Supported Sensors and Configuration

Sensors supported only with CM55-B0

| Sensor | CSI configuration | Parallel Configuration | Serial Configuration |
| --- | --- | --- | --- |
| OV2C | - SDR: 480x270 3FPS 160Mbps per lane, lane count = 1 - SDR: 1920x1080 60FPS 1.5Gbps per lane, lane count = 1 |  |  |
| OV5647 | - SDR: 480x270 30FPS 280Mbps per lane, lane count = 2 - SDR: 640x480 60FPS 280Mbps per lane, lane count = 2 |  |  |
| K351 | SDR: Serial Camera 500x500 4FPS |  |  |

# Supported Flash List

All supported with CM55-B0 and CM52.

| Vendor | Flash Part Number |
| --- | --- |
| Macronix | MX25U12843G |
| GigaDevice | GD25LE128E |
| WINBOND | W25Q128JW |

# Known Issues

| Issue | Summary | Components | Severity |
| --- | --- | --- | --- |
| SRSDKI-1514 | SDIO sample app failure is observed (SR110) | SDIO | High |
| SRSDKI-1835 | SPI sample app is not working - no logs printed on the logger (SR110) | SPI | High |
| SRSDKI-1884 | USB CDC connection error observed for Doorbell use case | Jpeg preroll | High |
| SRSDKI-1834 | FID HDG use case fails due to Model init failure | Vision | High |
| SRSDKI-1524 | I3C failure due to incorrect DAA configuration in non-loopback mode | I3C | High |
| SRSDKI-1526 | Pin connection details are missing in the readme file for I2S/TDM sample app | I2S/TDM | Medium |
| SRSDKI-1529 | Timeout is observed during SPI data transfers in non-loopback mode | SPI | Medium |
| SRSDKI-1530 | In SPI Pre-roll case, initial transfer passes, but subsequent transfers fail | SPI Pre-roll | Medium |
| SRSDKI-1335 | Current reading in Gear 0 mode is high in Doorbell Application | Jpeg preroll | Medium |

# Revision History

| Last Modified | Revision | Description |
| --- | --- | --- |
| June 2025 | 0.1 | Astra MCU SDK Release 1.0.0 |
| August 2025 | 0.2 | Astra MCU SDK Release 1.1.0 |
| November 2025 | 0.3 | Astra MCU SDK Release 1.2.0 |
| January 2026 | 0.4 | Astra MCU SDK Release 1.3.0 |
| April 2026 | 0.5 | Astra MCU SDK Release 1.4.0 |
| May 2026 | 0.6 | Astra MCU SDK Release 1.5.0 |
| August 2026 | 0.7 | Astra MCU SDK Release 1.6.0 |

Copyright

Copyright © 2024 Synaptics Incorporated. All Rights Reserved.

Trademarks

Synaptics; the Synaptics logo; are trademarks or registered trademarks of Synaptics Incorporated in the United States and/or other countries. All other trademarks are the property of their respective owners.

Notice

This document contains information that is proprietary to Synaptics Incorporated (“Synaptics”). The holder of this document shall treat all information contained herein as confidential, shall use the information only for its intended purpose, and shall not duplicate, disclose, or disseminate any of this information in any manner unless Synaptics has otherwise provided express, written permission.

Use of the materials may require a license of intellectual property from a third party or from Synaptics. This document conveys no express or implied licenses to any intellectual property rights belonging to Synaptics or any other party. Synaptics may, from time to time and at its sole option, update the information contained in this document without notice.

INFORMATION CONTAINED IN THIS DOCUMENT IS PROVIDED "AS-IS,” AND SYNAPTICS HEREBY DISCLAIMS ALL EXPRESS OR IMPLIED

WARRANTIES, INCLUDING BUT NOT LIMITED TO ANY IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE, AND ANY WARRANTIES OF NON-INFRINGEMENT OF ANY INTELLECTUAL PROPERTY RIGHTS.  IN NO EVENT SHALL SYNAPTICS BE

LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, PUNITIVE, OR CONSEQUENTIAL DAMAGES ARISING OUT OF OR IN CONNECTION

WITH THE USE OF THE INFORMATION CONTAINED IN THIS DOCUMENT, HOWEVER CAUSED AND BASED ON ANY THEORY OF LIABILITY,

WHETHER IN AN ACTION OF CONTRACT, NEGLIGENCE OR OTHER TORTIOUS ACTION, AND EVEN IF SYNAPTICS WAS ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.  IF A TRIBUNAL OF COMPETENT JURISDICTION DOES NOT PERMIT THE DISCLAIMER OF DIRECT DAMAGES OR ANY OTHER DAMAGES, SYNAPTICS’ TOTAL CUMULATIVE LIABILITY TO ANY PARTY SHALL NOT EXCEED ONE HUNDRED U.S. DOLLARS.

Ordering Information

For ordering information and a complete list of Synaptics' products, contact your Synaptics sales representative. Visit our website at www.synaptics.com to locate the Synaptics office nearest you.

Contact Us

Visit our website at www.synaptics.com to locate the Synaptics office nearest you.